Digital Logic Design

Below are the proper explanations of all 14 topics from your syllabus. I’m keeping the explanations detailed enough for understanding and exams, while using straightforward student-level language.

1. Number Systems

1.1 What is a Number System?

A number system is a method of representing numbers using a specific set of digits and a particular base (or radix).

The base tells us how many different digits are available.

For example:

- Decimal → base 10 → digits 0–9

- Binary → base 2 → digits 0 and 1

- Octal → base 8 → digits 0–7

- Hexadecimal → base 16 → digits 0–9 and A–F

Digital computers mainly use binary because electronic circuits can naturally represent two states:

- 0 → OFF/LOW

- 1 → ON/HIGH

1.2 Decimal Number System

The decimal system is base 10 and is the number system humans normally use.

Each position has a power of 10.

For example:

(425)10(425)_{10}

can be expanded as:

4(102)+2(101)+5(100)4(10^2)+2(10^1)+5(10^0) =400+20+5=425=400+20+5=425

For fractional numbers, powers become negative:

(25.36)10(25.36)_{10} =2(101)+5(100)+3(10−1)+6(10−2)=2(10^1)+5(10^0)+3(10^{-1})+6(10^{-2})

1.3 Binary Number System

Binary is base 2, so it only uses:

0,  10,\;1

Each binary digit is called a bit.

Example:

(1011)2(1011)_2

Its decimal value is:

1(23)+0(22)+1(21)+1(20)1(2^3)+0(2^2)+1(2^1)+1(2^0) =8+0+2+1=11=8+0+2+1=11

Therefore:

(1011)2=(11)10(1011)_2=(11)_{10}

Why binary is important

Digital circuits work with two electrical states, so binary is the natural language of digital systems.

1.4 Octal Number System

Octal is base 8.

Digits are:

0,1,2,3,4,5,6,70,1,2,3,4,5,6,7

Octal is useful because one octal digit represents exactly 3 binary bits.

Binary to Octal

Take:

10110110210110110_2

Group from the right into sets of 3:

010  110  110010\;110\;110

Convert each group:

- 010 = 2

- 110 = 6

- 110 = 6

Therefore:

(10110110)2=(266)8(10110110)_2=(266)_8

1.5 Hexadecimal Number System

Hexadecimal is base 16.

Its digits are:

0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F

where:

- A = 10

- B = 11

- C = 12

- D = 13

- E = 14

- F = 15

One hexadecimal digit represents 4 binary bits.

Example:

10101111210101111_2

Group into four:

1010  11111010\;1111

Then:

- 1010 = A

- 1111 = F

So:

(10101111)2=(AF)16(10101111)_2=(AF)_{16}

Why hexadecimal is commonly used

Binary numbers can become very long. Hexadecimal provides a compact representation.

For example:

11111100101010111111110010101011

is much easier to write as:

FCABF CAB

after grouping appropriately.

1.6 Base Conversion

Decimal to Binary

For an integer, repeatedly divide by 2 and record the remainders.

Example: Convert 13 to binary.

13÷2=6  R113\div2=6\;R1 6÷2=3  R06\div2=3\;R0 3÷2=1  R13\div2=1\;R1 1÷2=0  R11\div2=0\;R1

Read remainders from bottom to top:

(13)10=(1101)2(13)_{10}=(1101)_2

Binary to Decimal

Multiply each bit by its positional power.

Example:

110121101_2 =1(23)+1(22)+0(21)+1(20)=1(2^3)+1(2^2)+0(2^1)+1(2^0) =8+4+0+1=13=8+4+0+1=13

Binary to Hexadecimal

Group binary digits into groups of four from the right.

Example:

1101011011010110 1101  01101101\;0110 D  6D\;6

Therefore:

110101102=D61611010110_2=D6_{16}

Binary to Octal

Group binary digits into groups of three.

Example:

1101011011010110

Add leading zeros:

011  010  110011\;010\;110 3  2  63\;2\;6

Therefore:

110101102=326811010110_2=326_8

1.7 Signed Number Representation

Computers need to represent both positive and negative numbers.

Three common methods are:

- Signed magnitude

- 1's complement

- 2's complement

Signed Magnitude

The MSB (Most Significant Bit) is used as the sign bit.

- 0 → positive

- 1 → negative

For an 8-bit number:

00000101=+500000101=+5 10000101=−510000101=-5

The remaining 7 bits represent the magnitude.

Disadvantage

There are two representations of zero:

00000000=+000000000=+0 10000000=−010000000=-0

1.8 1's Complement

To obtain the negative of a binary number, invert every bit.

Example:

+5=00000101+5=00000101

Invert all bits:

1111101011111010

Therefore:

−5=11111010-5=11111010

Again, 1's complement has two representations of zero.

1.9 2's Complement

To find the 2's complement:

- Take the 1's complement.

- Add 1.

Example:

+5=00000101+5=00000101

1's complement:

1111101011111010

Add 1:

1111101111111011

Therefore:

−5=11111011-5=11111011

Why 2's complement is preferred

It provides only one zero representation and makes subtraction easier because subtraction can be performed using addition.

2. Logic Gates

Logic gates are the fundamental building blocks of digital circuits.

They perform Boolean operations on binary inputs.

The main gates are:

- AND

- OR

- NOT

- NAND

- NOR

- XOR

- XNOR

2.1 AND Gate

The AND gate produces 1 only when all inputs are 1.

For two inputs:

Y=A⋅BY=A\cdot B

Truth table:

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Example

If:

A=1,B=0A=1,\quad B=0

then:

Y=1⋅0=0Y=1\cdot0=0

2.2 OR Gate

The OR gate produces 1 when at least one input is 1.

Y=A+BY=A+B

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

2.3 NOT Gate

A NOT gate has one input and reverses it.

Y=A′Y=A'

| A | Y |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

It is also called an inverter.

2.4 NAND Gate

NAND means NOT AND.

First perform AND, then invert the result.

Y=(AB)′Y=(AB)'

The output is 0 only when all inputs are 1.

2.5 NOR Gate

NOR means NOT OR.

Y=(A+B)′Y=(A+B)'

The output is 1 only when all inputs are 0.

2.6 XOR Gate

XOR means Exclusive OR.

For two inputs, the output is 1 when the inputs are different.

Y=A⊕BY=A\oplus B

| A | B | Y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

XOR is heavily used in adders, parity circuits and comparison circuits.

2.7 XNOR Gate

XNOR is the complement of XOR.

Output is 1 when both inputs are the same.

Y=(A⊕B)′Y=(A\oplus B)'

It is therefore also called an equality detector.

2.8 Universal Gates

NAND and NOR are universal gates.

This means that any Boolean function can be implemented using only NAND gates or only NOR gates.

For example, NOT can be made using NAND:

A NAND A=A′A\ NAND\ A=A'

Similarly:

A NOR A=A′A\ NOR\ A=A'

This is an important examination concept.

2.9 TTL vs CMOS

TTL

TTL means Transistor-Transistor Logic.

It traditionally uses bipolar transistors.

Characteristics:

- Fast for many classic logic families

- Higher power consumption

- Commonly associated with 74-series logic

CMOS

CMOS means Complementary Metal-Oxide-Semiconductor.

Characteristics:

- Very low static power consumption

- High noise immunity

- High density

- Very common in modern digital systems

2.10 Propagation Delay

When an input changes, the output does not change exactly at the same instant.

The small delay is called propagation delay.

It affects the maximum speed of a circuit.

2.11 Fan-Out

Fan-out is the maximum number of standard inputs that one gate output can drive while maintaining proper operation.

3. Boolean Algebra

Boolean algebra is a mathematical system used to analyze and simplify digital logic.

Boolean variables can only have:

0 or 10\text{ or }1

The main operations are:

- AND → multiplication or dot

- OR → +

- NOT → complement

3.1 Important Boolean Laws

Identity Law

A+0=AA+0=A A⋅1=AA\cdot1=A

Null/Dominance Law

A+1=1A+1=1 A⋅0=0A\cdot0=0

Idempotent Law

A+A=AA+A=A A⋅A=AA\cdot A=A

Complement Law

A+A′=1A+A'=1 A⋅A′=0A\cdot A'=0

Involution Law

(A′)′=A(A')'=A

Commutative Law

A+B=B+AA+B=B+A AB=BAAB=BA

Associative Law

A+(B+C)=(A+B)+CA+(B+C)=(A+B)+C A(BC)=(AB)CA(BC)=(AB)C

Distributive Law

A(B+C)=AB+ACA(B+C)=AB+AC

Also:

A+BC=(A+B)(A+C)A+BC=(A+B)(A+C)

The second form is particularly important in Boolean algebra.

3.2 De Morgan's Theorems

These are among the most important Boolean laws.

First theorem

(AB)′=A′+B′(AB)'=A'+B'

Meaning:

Complement of AND = OR of complements

Second theorem

(A+B)′=A′B′(A+B)'=A'B'

Meaning:

Complement of OR = AND of complements

These theorems are extremely useful when converting between NAND/NOR implementations.

3.3 Principle of Duality

To obtain the dual of a Boolean expression:

- Replace AND with OR

- Replace OR with AND

- Replace 0 with 1

- Replace 1 with 0

For example:

Original:

A+0=AA+0=A

Dual:

A⋅1=AA\cdot1=A

Both are valid Boolean identities.

3.4 SOP — Sum of Products

SOP means Sum of Products.

It consists of AND terms connected by OR.

Example:

F=AB+A′C+BCF=AB+A'C+BC

Each product term is ANDed, and those terms are ORed together.

3.5 POS — Product of Sums

POS means Product of Sums.

It consists of OR terms connected through AND.

Example:

F=(A+B)(A′+C)(B+C)F=(A+B)(A'+C)(B+C)

3.6 Minterms

A minterm is a product term containing every variable exactly once, either complemented or uncomplemented.

For two variables:

A′B′,  A′B,  AB′,  ABA'B',\;A'B,\;AB',\;AB

A minterm corresponds to exactly one row of a truth table where the function is 1.

3.7 Maxterms

A maxterm is a sum term containing every variable exactly once.

Each maxterm corresponds to one truth-table row where the function is 0.

4. Combinational Logic Circuits

A combinational circuit is a digital circuit in which the output depends only on the present input values.

It does not have memory.

Examples include:

- Adders

- Subtractors

- Multiplexers

- Demultiplexers

- Decoders

- Encoders

- Comparators

4.1 Combinational Circuit Design Procedure

A common design process is:

Step 1: Understand the specification

Determine what the circuit should do.

Step 2: Identify inputs and outputs

Assign variables to all inputs and outputs.

Step 3: Create the truth table

List every possible input combination and determine the required output.

Step 4: Derive Boolean expressions

Use the truth table to obtain equations.

Step 5: Simplify

Use Boolean algebra or K-Map.

Step 6: Draw the circuit

Implement the simplified equations using gates.

Step 7: Verify

Check the circuit against the original truth table.

4.2 Multiplexer

A Multiplexer (MUX) is a data selector.

It selects one input out of many and sends it to a single output.

A 4-to-1 MUX has:

- 4 data inputs

- 2 select lines

- 1 output

Because:

22=42^2=4

In general:

2n inputs require n select lines2^n \text{ inputs require } n \text{ select lines}

Applications

- Data selection

- Communication systems

- CPU datapaths

- Implementing Boolean functions

4.3 Demultiplexer

A DEMUX performs the opposite operation.

It takes one input and routes it to one selected output.

For example, a 1-to-4 DEMUX has:

- 1 data input

- 2 select lines

- 4 outputs

Only one output receives the input at a time.

4.4 Decoder

A decoder converts an n-bit binary input into up to 2n2^n output lines.

Example:

3-to-8 Decoder

3 input bits produce:

23=82^3=8

possible outputs.

Typically, exactly one output is active for each input combination, so this is often described as a one-hot output.

Applications

- Memory address decoding

- Instruction decoding

- Display systems

- Control circuits

4.5 Encoder

An encoder performs the reverse operation of a decoder.

It converts one of many active inputs into a binary code.

Example:

8-to-3 Encoder

8 possible inputs are converted into 3 binary output bits.

A priority encoder handles situations where more than one input may be active by assigning priority to one input.

4.6 Comparator

A digital comparator compares two binary numbers.

For two numbers A and B, it usually produces three outputs:

- A>BA>B

- A=BA=B

- A<BA<B

Comparators are used in:

- Processors

- Counters

- Control systems

- Digital measurement systems

5. Simplification Methods — K-Map and Quine-McCluskey

The purpose of Boolean simplification is to reduce:

- Number of gates

- Number of inputs

- Cost

- Power consumption

- Circuit complexity

- Propagation delay

5.1 Karnaugh Map

A Karnaugh Map, or K-Map, is a graphical technique for simplifying Boolean functions.

It is mainly used for functions with a relatively small number of variables.

Common forms include:

- 2-variable

- 3-variable

- 4-variable

- 5-variable

- 6-variable

5.2 Gray Code Ordering

The K-Map does not use ordinary binary order.

It uses Gray code ordering, where adjacent cells differ in only one bit.

For four positions:

00,  01,  11,  1000,\;01,\;11,\;10

This arrangement allows adjacent cells to be combined.

5.3 K-Map Grouping Rules

For SOP simplification:

- Group 1s.

- Groups must contain 1,2,4,8,…1,2,4,8,\ldots cells.

- Use the largest possible groups.

- Groups may wrap around edges.

- Overlapping is allowed.

- Every required 1 must be covered.

For POS simplification:

- Group 0s instead.

5.4 Don't-Care Conditions

A don't-care condition is represented by:

XX

It means the output may be treated as either 0 or 1.

The designer can use it to make larger groups and simplify the circuit further.

Don't-cares are especially useful when certain input combinations never occur.

5.5 Prime Implicant

A prime implicant is a group that cannot be expanded further without including an unwanted value.

An essential prime implicant is a prime implicant that covers at least one required minterm not covered by any other prime implicant.

Essential prime implicants must be included in the final expression.

5.6 Quine-McCluskey Method

The Quine-McCluskey method is a systematic tabular method for Boolean minimization.

It is especially useful when the number of variables becomes too large for convenient K-Maps.

Basic procedure

Step 1: Write minterms in binary

Example:

m(1,3,5,7)m(1,3,5,7)

Convert each into binary.

Step 2: Group according to number of 1s

Terms containing the same number of 1s are placed in the same group.

Step 3: Compare adjacent groups

If two terms differ in only one bit, combine them.

The differing bit is replaced with a dash.

Example:

01010101

and

01110111

become:

01−101-1

Step 4: Repeat combination

Continue combining wherever possible.

Step 5: Find Prime Implicants

Terms that cannot be combined further are prime implicants.

Step 6: Prime Implicant Chart

Create a chart showing which prime implicants cover which minterms.

Step 7: Select Essential Prime Implicants

Choose essential terms first, then cover remaining minterms with the minimum number of terms.

6. Flip-Flops and Latches

Sequential circuits differ from combinational circuits because they possess memory.

A memory element stores one bit of information.

The two major storage devices are:

- Latches

- Flip-flops

6.1 Latch

A latch is generally level-triggered.

Its output can change while the enable/control condition is active.

Therefore, a latch is sometimes called transparent.

6.2 SR Latch

SR means:

- S = Set

- R = Reset

For an active-high NOR-based SR latch:

| S | R | Q(next) |
| --- | --- | --- |
| 0 | 0 | Previous state |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Invalid |

The invalid state occurs because both set and reset are activated simultaneously.

6.3 D Latch

The D latch solves the invalid-state problem associated with the basic SR latch.

D stands for Data.

When enable is active:

Q=DQ=D

When enable is inactive:

QQ

retains its previous value.

This makes D latches useful for data storage.

6.4 Flip-Flop

A flip-flop is generally edge-triggered.

It changes state only at a clock transition:

- Rising edge: 0 → 1

- Falling edge: 1 → 0

This makes timing much more controlled than a transparent latch.

6.5 SR Flip-Flop

It has Set and Reset inputs.

Its major problem remains the forbidden state in some implementations.

6.6 D Flip-Flop

A D flip-flop captures the data input at the active clock edge.

Characteristic equation:

Qnext=DQ_{next}=D

It is extensively used in:

- Registers

- Shift registers

- Memory

- Pipelines

6.7 JK Flip-Flop

The JK flip-flop improves the SR flip-flop.

| J | K | Q(next) |
| --- | --- | --- |
| 0 | 0 | Q |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Q' |

When:

J=K=1J=K=1

the output toggles.

Characteristic equation:

Qnext=JQ′+K′QQ_{next}=JQ'+K'Q

6.8 T Flip-Flop

T means Toggle.

| T | Q(next) |
| --- | --- |
| 0 | Q |
| 1 | Q' |

Characteristic equation:

Qnext=T⊕QQ_{next}=T\oplus Q

T flip-flops are widely used in counters.

6.9 Characteristic Table vs Excitation Table

Characteristic table

Shows the next state for given inputs and present state.

Excitation table

Shows what input values are required to move from the present state to the desired next state.

This distinction is important during sequential-circuit design.

7. Asynchronous and Synchronous Circuits

Sequential systems can operate in asynchronous or synchronous ways.

7.1 Asynchronous Circuits

In an asynchronous circuit, state changes are not controlled by one common global clock.

Changes can occur when inputs change.

Advantages

- No clock distribution required

- Potentially fast response

- Can reduce clock-related power

Disadvantages

- Timing is more difficult

- Hazards and glitches can occur

- Race conditions can occur

- Design and debugging are more difficult

7.2 Synchronous Circuits

A synchronous circuit uses a clock signal to coordinate state changes.

State transitions generally occur on a specified clock edge.

Advantages

- Predictable timing

- Easier design

- Easier testing

- Better synchronization

Disadvantages

- Clock distribution consumes power

- Circuit cannot normally change state outside the clocking scheme

- Maximum operating frequency is limited by timing constraints

7.3 Important Difference

| Synchronous | Asynchronous |
| --- | --- |
| Uses clock | Usually no global clock |
| More predictable | More timing-sensitive |
| Easier to design | More difficult |
| Common in digital systems | Used in specialized applications |
| State changes coordinated | State changes depend on events/inputs |

8. Counters and Shift Registers

8.1 Counter

A counter is a sequential circuit that goes through a predetermined sequence of states.

Counters are usually constructed using flip-flops.

8.2 Asynchronous/Ripple Counter

In a ripple counter:

- First flip-flop receives the external clock.

- The next flip-flop is triggered by the previous flip-flop.

- The clock effect therefore propagates through the chain.

Advantage

Simple hardware.

Disadvantage

Propagation delay accumulates.

Therefore ripple counters are slower.

8.3 Synchronous Counter

All flip-flops receive the same clock.

The required next-state behavior is achieved through combinational logic.

Advantage

Much faster than ripple counters because the flip-flops change in a coordinated way.

8.4 Up Counter

Counts upward:

000→001→010→011→100…000\rightarrow001\rightarrow010\rightarrow011\rightarrow100\ldots

8.5 Down Counter

Counts downward:

111→110→101→100…111\rightarrow110\rightarrow101\rightarrow100\ldots

8.6 Up/Down Counter

Can count either upward or downward depending on a control input.

8.7 Mod-N Counter

A Mod-N counter contains N distinct states.

For example, a Mod-8 counter has 8 states:

0 through 70\text{ through }7

Number of flip-flops required must satisfy:

2n≥N2^n\ge N

For Mod-10:

24=16≥102^4=16\ge10

So at least 4 flip-flops are required.

8.8 Shift Register

A shift register is a group of flip-flops used to store and shift binary data.

On each active clock edge, the stored data moves from one flip-flop to the next.

8.9 SISO

Serial In Serial Out

Data enters one bit at a time and also leaves one bit at a time.

Applications:

- Delay lines

- Serial data transfer

8.10 SIPO

Serial In Parallel Out

Data enters serially but becomes available simultaneously at several outputs.

Used for:

- Serial-to-parallel conversion

8.11 PISO

Parallel In Serial Out

Several bits are loaded simultaneously and then shifted out one by one.

Used for:

- Parallel-to-serial conversion

8.12 PIPO

Parallel In Parallel Out

Data is loaded and retrieved in parallel.

It is essentially a parallel storage register.

8.13 Ring Counter

A ring counter circulates a bit pattern, usually a single 1.

For example, with four flip-flops:

1000→0100→0010→0001→10001000\rightarrow0100\rightarrow0010\rightarrow0001\rightarrow1000

8.14 Johnson Counter

A Johnson counter feeds the complement of the final flip-flop output back into the first flip-flop.

An n-bit Johnson counter normally produces:

2n2n

different states.

For 4 flip-flops:

2(4)=82(4)=8

states.

9. Triggered Devices and Their Types

Triggering refers to when a sequential device responds to a control or clock signal.

9.1 Level Triggering

A level-triggered device responds for the duration of a particular clock or enable level.

For example:

- HIGH-level triggered

- LOW-level triggered

Latches are commonly level-sensitive.

When active, the output may follow the input.

9.2 Edge Triggering

An edge-triggered flip-flop responds at a transition rather than during the entire clock level.

Positive edge

0→10\rightarrow1

Negative edge

1→01\rightarrow0

The data is sampled around that clock edge.

9.3 Master-Slave Triggering

A master-slave flip-flop uses two stages:

- Master

- Slave

The two stages operate during opposite clock phases.

For example, while the master is active, the slave is inactive. Then the slave becomes active after the clock changes phase.

This prevents undesirable repeated changes and helps eliminate the race-around problem in JK flip-flops.

9.4 Race-Around Condition

In a level-triggered JK flip-flop, when:

J=K=1J=K=1

the output should toggle.

But if the clock pulse remains active long enough, the output can toggle repeatedly within the same pulse.

This is called the race-around condition.

It can be prevented by:

- Edge-triggered flip-flops

- Master-slave configuration

- Very short clock pulses

10. Binary Arithmetic and Arithmetic Circuits

Digital systems perform arithmetic using logic circuits.

10.1 Binary Addition

Binary addition follows these rules:

0+0=00+0=0 0+1=10+1=1 1+0=11+0=1 1+1=101+1=10

Here:

- Sum = 0

- Carry = 1

10.2 Half Adder

A half adder adds two binary bits:

Inputs:

- A

- B

Outputs:

- Sum

- Carry

The equations are:

S=A⊕BS=A\oplus B C=ABC=AB

Truth table:

| A | B | S | C |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

10.3 Full Adder

A full adder adds:

- A

- B

- Carry-in CinC_{in}

Outputs:

- Sum

- Carry-out CoutC_{out}

Sum equation:

S=A⊕B⊕CinS=A\oplus B\oplus C_{in}

Carry equation:

Cout=AB+Cin(A⊕B)C_{out}=AB+C_{in}(A\oplus B)

A full adder can also be constructed using two half adders and an OR gate.

10.4 Binary Subtraction

Subtraction can be implemented using 2's complement.

For:

A−BA-B

we perform:

A+(2’s complement of B)A+(\text{2's complement of }B)

Example:

7−37-3

Represent both using fixed-width binary.

Then take the 2's complement of 3 and add it to 7.

This allows the same adder hardware to perform both addition and subtraction.

10.5 Ripple Carry Adder

A ripple carry adder is created by connecting full adders in series.

The carry output of one stage becomes the carry input of the next.

Problem

The carry must travel through each stage.

Therefore:

More bits → more delay

10.6 Carry Lookahead Adder

The carry lookahead adder reduces carry propagation delay by calculating carries more directly.

For each bit:

Generate

Gi=AiBiG_i=A_iB_i

Propagate

Pi=Ai⊕BiP_i=A_i\oplus B_i

Carry is then derived using these signals.

For example:

Ci+1=Gi+PiCiC_{i+1}=G_i+P_iC_i

The main advantage is speed.

The disadvantage is more complex hardware.

10.7 Binary Multiplier

Binary multiplication is conceptually similar to decimal multiplication.

Partial products are generated using AND gates and then added.

Hardware multipliers can use:

- Shift-and-add methods

- Array multiplier structures

10.8 Arithmetic Logic Unit (ALU)

The ALU is a combinational component that performs arithmetic and logical operations.

Typical operations include:

Arithmetic

- Addition

- Subtraction

- Increment

- Decrement

Logic

- AND

- OR

- XOR

- NOT

- Comparisons

Control/select lines determine which operation is performed.

The ALU is a fundamental part of a CPU.

11. Memory Elements

Memory stores binary information.

Memory can broadly be classified as:

- Volatile

- Non-volatile

11.1 RAM

RAM stands for Random Access Memory.

It normally allows both reading and writing.

RAM is volatile, meaning its stored contents are generally lost when power is removed.

Two major types are:

- SRAM

- DRAM

11.2 SRAM

SRAM stands for Static RAM.

It stores each bit using a bistable circuit, commonly a flip-flop-like cell.

Characteristics

- Fast

- No periodic refresh required

- More expensive per bit

- Lower density than DRAM

SRAM is commonly used for cache memory.

11.3 DRAM

DRAM stands for Dynamic RAM.

A DRAM cell stores information as charge in a capacitor.

Because charge leaks over time, the memory requires periodic refreshing.

Characteristics

- High density

- Lower cost per bit

- Requires refresh

- Commonly used as main memory

11.4 ROM

ROM means Read-Only Memory.

It is generally non-volatile.

Several types exist.

Mask ROM

Programmed by the manufacturer.

The stored data cannot normally be changed by the user.

PROM

Programmable Read-Only Memory

The user can program it once.

After programming, it cannot normally be reprogrammed.

EPROM

Erasable Programmable ROM

It can be erased, traditionally using ultraviolet light, and then reprogrammed.

EEPROM

Electrically Erasable Programmable ROM

It can be electrically erased and reprogrammed.

It is useful for storing non-volatile configuration and firmware-related data.

11.5 Memory Organization

A memory chip contains several important signal types.

Address lines

Select the memory location.

For nn address lines:

2n2^n

different locations can be selected.

Data lines

Carry the actual data being read or written.

Chip Select

Activates the particular memory device.

Read/Write control

Determines whether information is being read or written.

12. Finite State Machines

A Finite State Machine (FSM) is a mathematical and hardware model used to design sequential systems.

An FSM consists of a finite number of states and transitions between them.

Examples include:

- Traffic-light controllers

- Vending machines

- Sequence detectors

- Communication controllers

- Control units

12.1 Main Components of an FSM

An FSM contains:

- Present state

- Inputs

- Next-state logic

- Memory elements

- Outputs

The memory elements, usually flip-flops, store the present state.

12.2 Moore Machine

In a Moore machine, the output depends only on the present state.

Output=f(State)Output=f(State)

Advantage

Outputs are generally more stable and naturally synchronized with state changes.

Disadvantage

Sometimes more states are required.

12.3 Mealy Machine

In a Mealy machine:

Output=f(State,Input)Output=f(State,Input)

Therefore the output depends on both:

- Present state

- Current input

Advantage

Often requires fewer states and can respond faster to an input.

Disadvantage

Because outputs depend directly on inputs, they can potentially experience glitches if inputs change asynchronously.

12.4 Moore vs Mealy

| Moore | Mealy |
| --- | --- |
| Output depends on state | Output depends on state and input |
| Usually more states | Often fewer states |
| Output changes with state | Output can change immediately with input |
| Easier to make stable | Potentially more sensitive to input changes |

12.5 FSM Design Procedure

Step 1: Understand the problem

Determine what the sequential circuit must do.

Step 2: Identify states

Create states representing the required conditions.

Step 3: Draw state diagram

Represent transitions using arrows.

Step 4: Create state table

List:

- Present state

- Inputs

- Next state

- Outputs

Step 5: Assign binary values to states

This is called state encoding.

Step 6: Select flip-flops

For example:

- D

- JK

- T

Step 7: Derive excitation equations

Use the flip-flop excitation table.

Step 8: Derive output equations

Determine the required output logic.

Step 9: Implement the circuit

Use gates and flip-flops.

Step 10: Verify

Check all possible transitions.

12.6 State Encoding

Common state encoding methods include:

Binary encoding

Uses the minimum number of flip-flops.

For N states:

⌈log⁡2N⌉\lceil\log_2N\rceil

flip-flops are generally required.

Gray encoding

Adjacent states differ by only one bit.

One-hot encoding

Uses one flip-flop per state.

It requires more flip-flops but can make logic simpler and is especially common in FPGA designs.

13. Programmable Logic Devices

Programmable Logic Devices are integrated circuits that users can configure to implement digital logic.

Examples include:

- PLA

- PAL

- CPLD

- FPGA

13.1 SPLD

SPLD means Simple Programmable Logic Device.

Examples:

- PAL

- PLA

These are useful for smaller logic functions.

13.2 PLA

PLA stands for Programmable Logic Array.

Both:

- AND plane

- OR plane

are programmable.

It provides flexibility for implementing Boolean expressions.

13.3 PAL

PAL stands for Programmable Array Logic.

Typically:

- AND plane is programmable

- OR plane is fixed or structured

PAL devices are generally simpler and faster than fully programmable PLA structures for many applications.

13.4 CPLD

CPLD stands for Complex Programmable Logic Device.

It consists of multiple programmable logic blocks connected through programmable interconnection resources.

Typical characteristics

- Good for medium-size logic

- Predictable timing

- Fast startup

- Often non-volatile configuration

- Useful for control logic, glue logic, decoders and state machines

13.5 FPGA

FPGA stands for Field-Programmable Gate Array.

An FPGA contains a large number of configurable resources.

Typical components include:

- Configurable Logic Blocks

- Look-Up Tables

- Flip-flops

- Programmable routing

- I/O blocks

- Often dedicated memory

- Often DSP resources

Applications

- Digital signal processing

- Image processing

- Hardware acceleration

- Communication systems

- Embedded systems

- ASIC prototyping

13.6 CPLD vs FPGA

| CPLD | FPGA |
| --- | --- |
| Lower/moderate capacity | Very high capacity |
| Predictable timing | More complex timing |
| Often non-volatile configuration | Commonly SRAM-based |
| Fast startup | Usually requires configuration at startup |
| Good for control logic | Good for complex digital systems |
| Smaller designs | Large designs |

A key exam point is that FPGA generally provides much greater logic capacity and specialized resources, while CPLDs are attractive for smaller control-oriented designs with predictable timing and fast startup.

14. Verilog/VHDL and MultiSim

This topic focuses on implementing and testing digital circuits using software tools.

14.1 Hardware Description Language

An HDL is a language used to describe digital hardware.

Two major HDLs are:

- Verilog

- VHDL

An HDL can describe:

- Combinational logic

- Sequential logic

- Modules

- Registers

- Finite state machines

- Complex digital systems

14.2 Verilog

Verilog has a syntax that is relatively compact and familiar to students with programming experience.

A simple AND gate can be represented conceptually as:

assign y = a & b;

HDLs are not simply software programs; they describe hardware structures and behavior that can be synthesized into physical logic.

14.3 VHDL

VHDL is a strongly typed hardware description language.

It uses concepts such as:

- Entity

- Architecture

- Signals

- Processes

It is widely used in many engineering and hardware-design environments.

14.4 RTL

RTL stands for Register Transfer Level.

It describes:

- Data movement between registers

- Combinational logic between registers

- Clocked behavior

RTL is a major abstraction level used before hardware synthesis.

14.5 Dataflow Modeling

Dataflow modeling describes how data moves through expressions.

In Verilog:

assign y = (a & b) | c;

This represents a continuous relationship between inputs and output.

14.6 Behavioral Modeling

Behavioral modeling describes what the circuit should do using procedural constructs.

Common constructs include:

- if/else

- case

- always blocks

It is particularly useful for:

- Sequential logic

- FSMs

- Complex control logic

14.7 Structural Modeling

Structural modeling describes a circuit by connecting smaller components together.

For example, a full adder could be built by instantiating:

- Two half adders

- One OR gate

This represents the actual structure of the hardware.

14.8 Testbench

A testbench is a simulation environment used to test a design.

It provides input stimuli and observes outputs.

For example, a testbench can test every possible combination of a 2-input logic gate.

A testbench is generally not synthesized as part of the hardware because its purpose is verification.

14.9 Simulation

Simulation allows designers to test a circuit before implementing it physically.

It can reveal:

- Logic errors

- Timing problems

- Incorrect state transitions

- Unexpected outputs

14.10 MultiSim

MultiSim is a schematic-based circuit simulation environment.

It allows students and engineers to virtually construct circuits using:

- Logic gates

- ICs

- Flip-flops

- Switches

- LEDs

- Probes

- Oscilloscopes

- Logic analyzers

Instead of physically wiring a circuit first, it can be tested virtually.

14.11 Transient Analysis

Transient analysis examines how signals change with time.

It is useful for observing:

- Clock pulses

- Propagation delays

- Switching behavior

- Glitches

- Timing relationships

14.12 Virtual Instruments

MultiSim and similar environments can provide virtual versions of laboratory equipment such as:

- Oscilloscope

- Multimeter

- Function generator

- Logic analyzer

This allows circuit behavior to be observed without immediately using physical hardware.

Important Exam Revision Map

For this subject, the topics that usually require the most actual problem-solving practice are:

Highest-priority numerical/design topics

Number systems → Boolean algebra → K-Maps → Quine-McCluskey → Flip-flops → Counters → Adders/Subtractors → FSM design

You should be able to solve, not just define, these areas.

Very important theory/comparison topics

Logic gates → combinational circuits → synchronous vs asynchronous → latches vs flip-flops → Moore vs Mealy → SRAM vs DRAM → CPLD vs FPGA → Verilog/VHDL concepts

Formulas worth memorizing

F=k∣q1q2∣r2F=\frac{k|q_1q_2|}{r^2}

Not applicable to DLD; focus instead on these DLD equations:

SHA=A⊕BS_{HA}=A\oplus B CHA=ABC_{HA}=AB SFA=A⊕B⊕CinS_{FA}=A\oplus B\oplus C_{in} Cout=AB+Cin(A⊕B)C_{out}=AB+C_{in}(A\oplus B) QD(next)=DQ_{D(next)}=D QT(next)=T⊕QQ_{T(next)}=T\oplus Q QJK(next)=JQ′+K′QQ_{JK(next)}=JQ'+K'Q 2n≥N2^n\ge N

for determining the number of flip-flops needed for an N-state counter.

One Very Important Concept to Remember

A common exam mistake is confusing combinational and sequential circuits.

Combinational:

Output=f(Current Inputs)Output=f(Current\ Inputs)

No memory.

Examples:

Adder, MUX, Decoder, Encoder, Comparator

Sequential:

Output=f(Current Inputs, Previous State)Output=f(Current\ Inputs,\ Previous\ State)

Has memory.

Examples:

Flip-flop, Register, Counter, FSM

That distinction connects a large portion of the entire DLD syllabus.